- Chemical Results from the April 2001 NPEO Project

Water sampling procedure

Modifications of the aircraft (Twin Otter) sampling approach were undertaken that afforded significant improvements in sampling conditions with respect to the operation in 2000. Upon landing the aircraft at a particular site, a gas-powered generator was set running in the aft storage compartment with its exhaust venting out the slightly ajar compartment door (Photo 1). A Hermann Nelson heater was plugged into this generator and set on the ice adjacent to the pilot's side of the aircraft. Its ducting was routed into the aircraft through the pilot's window (Photo 2). The motor head of the ice drill was held in the heat stream of the Hermann Nelson while snow was shoveled from the ice in the vicinity of the main cargo doors of the aircraft. The warmed drill motor was then used to drill a 10-inch diameter hole through the ice. Next a nylon shelter was suspended from the cargo doors and a gantry for a small hydrographic winch set into place (Photo 1). The combination of the waste heat of the generator from the rear compartment and the heat from the Hermann Nelson from the forward section kept the aircraft and area within the nylon shelter warm enough to prevent freezing of the samples, CTD, XCTD equipment (and personnel!).

- Photo 1: View of the aft Twin Otter aircraft set-up for hydrographic sampling, April, 2001.

Photo 2: View of the forward Twin Otter aircraft set-up for hydrographic sampling, April, 2001. Co-Pilot Kevin Heslop placing ducting from Hermann Nelson heating unit through cockpit window.

Seawater samples were obtained using the battery-powered portable winch to cast 4 1.5-L Niskins at a time. The custom constructed Niskins (General Oceanics model 1010 with 8" extension and modified plunger) were mounted on a small diameter Kevlar line, with the deepest one mounted 1 meter above an internally recording Seacat SBE19 CTD. These were lowered to target depth as indicated on the winch meter wheel. Before the wire was paid back in, a slotted PVC disc connected to a vacuum pump was slipped over the wire and set on the ice to cover the hole. This system, provided by Dr. Humphrey Melling of the Institute of Ocean Sciences, Sidney, BC, prevents build up of ice on the line and allows the line to re-spool smoothly (see lower right Photo 3). Of course this must be removed from the wire just before each Niskin and the CTD are retrieved.

Photo 3: Jamie Morison of APL-UW retrieving a Niskin bottle in the sampling shelter at the forward cargo doors of the Twin Otter aircraft, April, 2001.

Upon retrieval, each Niskin was transferred to a rack in the heated aircraft and samples were drawn immediately. Salinity samples were collected into 125-ml amber glass bottles the caps of which were fitted with conical polyethylene inserts. Oxygen isotope samples were collected into similar containers of smaller volume (20-ml). Barium samples were collected into 20-ml polyethylene scintillation vials. Nutrient samples were collected into 60-ml polyethylene bottles and stored frozen until analysis. At the North Pole camp station, samples were also drawn into calibrated flasks for O2 (dissolved oxygen gas). These samples were taken first and in replicate for the purposes of testing conditions for calibrating an O2-sensor to be deployed in 2002. The usual pickling reagents (MnCl2 and NaOH/NaI) were added to these samples immediately upon collection following standard procedures (Dickson, 1994).

- Table 1: North Pole Environmental Observatory CTD station data

| CTD Cast No. | Station ID | Latitude | Longitude | Date | Time UTC |
| --- | --- | --- | --- | --- | --- |
| Cast 2 | 2001 Station 1; camp | 89° 34.2' N | 73° 03.5' E | 4/9/2001 | 17:38 |
| Cast 3* | 2000 Station 4 | 87° 28.4' N | 90° 50.2' W | 4/9/2001 | 21:18 |
| Cast 4* | 2000 Station 6 | 84° 55.4' N | 67° 23.4' W | 4/10/2001 | 13:52 |
| Cast 5* | 2000 Station 1 | 89° 38.3' N | 166° 44.5' W | 4/11/2001 | 18:49 |
| Cast 6 | 2001 Station 3 | 87° 14.7' N | 179° 48.5' E | 4/12/2001 | 08:53 |
| Cast 7 | 2001 Station 2 | 88° 59.2' N | 179° 41.4' W | 4/12/2001 | 22:42 |
| Cast 8 | 2001 Station 5 | 85° 01.3' N | 166° 28.9' W | 4/13/2001 | 16:56 |
| Cast 9 | 2001 Station 4 | 86° 08.2' N | 171° 02.0' W | 4/13/2001 | 20:03 |
| Cast 10 | 2001 Station 1;camp | 89° 28.3' N | 54° 43.2' E | 4/14/2001 | 13:02 |

*reoccupation of 2000 stations-CTD casts only, no water samples taken

- Analytical procedures

Bottle salinities were analyzed using a Guildline Autosal standardized with IAPSO standard seawater. Precision for these determinations was 0.002 practical salinity units. Analyses of the frozen phosphate, silicic acid, nitrate, nitrite and ammonia samples were performed ashore using a hybrid Alpkem RFA 300 and Technicon AA-II (AutoAnalyzer II) — based system and the JGOFS/WOCE suggested nutrient protocols (Gordon et al., 1994). The silicic acid, nitrate plus nitrite and nitrite channels were RFA-based, the phosphate and ammonium channels, AA-II. The samples were thawed and immediately analyzed for all nutrients. After standing overnight in the dark, the samples were reanalyzed for silicic acid to avoid polymerization effects (Gordon et al., 1994). The short-term precision of the nutrient analyses is typically: Silicic acid, 0.2%; phosphate, 0.4%; nitrate 0.3%; nitrite, 0.02µM; and ammonium, 0.03µM. However, the inter-cruise reproducibility achieved during the WOCE Hydrographic Program, Pacific One-Time Survey was for silicic acid, phosphate and nitrate, respectively, ca. 1%, 2% and 1% (unpublished data, Ross et al., 2000).

Barium was determined by isotope-dilution using an VG Thermo Excel inductively coupled quadrupole mass spectrometer as previously described with minor modifications (Falkner et al., 1994). Precision is estimated to be 3% for this suite of Ba measurements. Oxygen isotopes were analyzed by the CO2 equilibration method on the COAS Finnegan Mat 251. Results are reported in del units relative to VSMOW and 1-sigma precision is estimated to be +/- 0.05. Dissolved O2 was analyzed by modified Winkler titration to a starch endpoint (Dickson, 1994) at Alert within 24 hours of sample collection. Replicates were returned to Oregon State University and analyzed 14 days later. Care was taken to assure that the ground glass joint remained moist throughout sample transport and storage. In the laboratory, dissolved oxygen was measured by whole-flask titrations and amperometric end-point detection using a PC-controlled titrator (Culberson and Huang, 1987). The actual apparatus followed the adaptation to PC's by Knapp et al. (1989).

- Results

Data are given in Table 1. The CTD depths and temperatures were assigned to the individual bottles by matching the bottle salinity to the CTD salinity at the depth closest to the target value. The depth values given in the table have been rounded to the nearest tenth meter in the surface sample and nearest meter at depth. Temperatures are for the specific matching sample. One chemistry and salinity of a sample obtained from 500 m at station 2 (not shown), showed the Niskin did not trip until near the surface. CTD information for the upper portion of the cast at station 2 appears to be unreliable and so is not given. Otherwise depth matching was straightforward since salinities agreed within expected analytical error. There was no evidence of sample freezing and so unlike in 2000, no corrections to the analytical data are required.

Table 1 (panel A)

| Date | Station | target depth m | CTD m | bot sal psu | CTD temp degC | P µM | NO3 µM | NO2 µM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14-Apr-01 | 1 | 5 | 9.9 | 32.284 | -1.77 | 0.384 | 2.69 | 0.014 |
| 14-Apr-01 | 1 | 100 | 101 | 34.135 | -1.55 | 0.665 | 7.62 | 0.011 |
| 14-Apr-01 | 1 | 275 | 272 | 34.863 | 1.43 | 0.873 | 12.26 | 0.006 |
| 14-Apr-01 | 1 | 500 | 510 | 34.881 | 0.70 | 0.887 | 12.45 | 0.006 |
| 12-Apr-01 | 2 | 5 | 13.5 | 31.642 | -1.74 | 0.426 | 2.27 | 0.006 |
| 12-Apr-01 | 2 | 100 | 99 | 33.885 | -1.71 | 0.663 | 7.11 | 0.006 |
| 12-Apr-01 | 2 | 275 | 276 | 34.835 | 0.91 | 0.880 | 12.52 | 0.006 |
| 12-Apr-01 | 3 | 5 | - | 31.711 | - | 0.523 | 2.21 | 0.008 |
| 12-Apr-01 | 3 | 100 | 104 | 34.010 | -1.57 | 0.676 | 7.90 | 0.008 |
| 12-Apr-01 | 3 | 275 | 276 | 34.846 | 1.06 | 0.873 | 12.44 | 0.011 |
| 13-Apr-01 | 4 | 5 | 11 | 31.409 | -1.73 | 0.491 | 2.21 | 0.008 |
| 13-Apr-01 | 4 | 75 | 75 | 33.382 | -1.75 | 0.646 | 6.20 | 0.008 |
| 13-Apr-01 | 4 | 100 | 99 | 33.934 | -1.62 | 0.648 | 7.49 | 0.005 |
| 13-Apr-01 | 4 | 275 | 276 | 34.851 | 1.11 | 0.865 | 12.50 | 0.005 |
| 13-Apr-01 | 4 | 500 | 505 | 34.869 | 0.52 | 0.889 | 12.85 | 0.005 |
| 13-Apr-01 | 5 | 5 | 3 | 31.144 | -1.73 | 0.809 | 1.29 | 0.018 |
| 13-Apr-01 | 5 | 75 | 75 | 33.427 | -1.76 | 0.719 | 6.74 | 0.010 |
| 13-Apr-01 | 5 | 275 | 276 | 34.835 | 0.86 | 0.867 | 12.60 | 0.013 |
| 13-Apr-01 | 5 | 500 | 512 | 34.869 | 0.47 | 0.897 | 12.95 | 0.006 |

Table 1 (panel B)

| Station | target depth m | % Pacific water* | Si µM | del18O | Ba nM | O2 ml/L starch | O2 ml/L amperometric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 10.0 | 4.96 | -2.83 | 56.3 | 8.901 | 8.767 |
| 1 | 100 | 8.6 | 4.97 | -0.53 | 45.1 | 7.980 | 7.902 |
| 1 | 275 | nd | 6.01 | 0.23 | 43.3 | 7.354 | 7.448 |
| 1 | 500 | nd | 6.79 | 0.19 | 42.3 | 7.415 | 7.378 |
| 2 | 5 | 21.9 | 5.78 | -3.48 | 58.0 | nd | nd |
| 2 | 100 | 13.0 | 6.56 | -0.88 | 48.2 | nd | nd |
| 2 | 275 | nd | 7.08 | 0.25 | 43.4 | nd | nd |
| 3 | 5 | 38.1 | 6.33 | -3.75 | 69.5 | nd | nd |
| 3 | 100 | 7.7 | 5.57 | -0.54 | 49.5 | nd | nd |
| 3 | 275 | nd | 6.35 | 0.24 | 45.4 | nd | nd |
| 4 | 5 | 33.1 | 6.38 | -3.65 | 61.3 | nd | nd |
| 4 | 75 | 18.7 | 7.16 | -1.13 | 53.0 | nd | nd |
| 4 | 100 | 7.1 | 5.65 | -0.72 | 47.8 | nd | nd |
| 4 | 275 | nd | 6.44 | 0.22 | 43.9 | nd | nd |
| 4 | 500 | nd | 6.95 | 0.18 | 43.7 | nd | nd |
| 5 | 5 | 84.0 | 8.77 | -3.45 | 66.6 | nd | nd |
| 5 | 75 | 24.7 | 9.04 | -1.44 | 50.1 | nd | nd |
| 5 | 275 | nd | 6.75 | 0.21 | 41.6 | nd | nd |
| 5 | 500 | nd | 7.30 | 0.21 | 42.9 | nd | nd |

*see discussion for derivation of this parameter; nd = not done

References

Culbertson, C.H. and S. Huang, Automated amperometric oxygen titration, Deep-Sea Res., 34: 875-880, 1987.

Dickson, A. G., Determination of dissolved oxygen in sea water by Winkler titration, WOCE Hydrographic Program, Operations and Methods Manual, pp. 11 loose-leaf, Woods Hole, Mass., 1994.

Falkner, K.K., R.W. MacDonald, E.C. Carmack, and T. Weingartner, The potential of barium as a tracer of Arctic water masses, in The Polar Oceans and Their Role in Shaping the Global Environment: The Nansen Centennial Volume, AGU Geophys. Monograph Series, edited by O.M. Johannessen, R.D. Muench, and J.E. Overland, pp. 63-76, AGU Books, Washington DC, 1994.

Gordon, L.I., J.J.C. Jr., A.A. Ross, and J.M. Krest, A suggested protocol for continuous flow automated analysis of seawater nutrients (phosphate, nitrate, nitrite and silicic acid) in the WOCE Hydrographic Program and the Joint Global Ocean Fluxes Study. WOCE Operations Manual, Vol. 3: The Observational Program, Section 3.1: WOCE Hydrographic Program, Part 3.1.3: WHP Operations and Methods., pp. 52 loose-leaf, Woods Hole, Mass., 1994.

Knapp, GP, MC Stalcup and RJ Stanley, Dissolved oxygen measurements in sea water at the Woods Hole Oceanographic Institution, Technical Report, WHOI-89-23, Woods Hole Oceanographic Institution, Woods Hole, MA 02543. 13 pp., 1989.

Acknowledgements

K. Falkner wishes to acknowledge laboratory assistance from Chi Meredith, Andy Ross and Joe Jennings of OSU and CTD data assistance from Roger Anderson of APL-UW.  The First Air crew, consisting of pilot Paul Rask, co-pilot Kevin Heslop and engineer Doug Ivemy, contributed well above and beyond the call of duty to the sampling effort.  Colleagues Andy Heiberg, Jamie Morison, Sigrid Salo and Dean Stewart are thanked for their help and comradery in the field.  Financial support was provided by the National Science Foundation under grant OPP-9910335.  Any opinions, findings, and conclusions or recommendations expressed in this material are those of the author(s) and do not necessarily reflect the views of the National Science Foundation.